ROTUNDA STAGE

TECHNICAL SPECS

TABLE OF CONTENTS

PRODUCTION CONTACTS	3

ROOM OVERVIEW	3

LOADING AREA	3

PARKING	3

STAGE DOOR	4

HOUSE SOFTGOODS	4

STAGE EQUIPMENT	4

LIGHTING SYSTEM OVERVIEW	5

LIGHTING INVENTORY	6

SOUND SYSTEM OVERVIEW	7

HOUSE SPEAKER SYSTEM	7

AUDIO INVENTORY	8

VIDEO / STREAMING OVERVIEW	11

VIDEO/STREAMING INVENTORY	11

GROUNDPLAN 	12

# PRODUCTION CONTACTS

Director of Production – John Penisten: 608-258-4451 jpenisten@overture.org

Technical Coordinator – Karen Corbeill: 608-258-4437 kcorbeill@overture.org

Sound Supervisor – Patrick Bley: 608-982-9799 pbley@overture.org

Lighting Supervisor – Zak Stowe: 608-258-4961 zstowe@overture.org

Video Supervisor – Joseph Garcia-Menocal: 608-258-4171 jgarciamenocal@overture.org

# ROOM OVERVIEW

SEATING

Amphitheater style seating: 200
Maximum capacity (with additional chairs added):	300

Various layout options exist for table seating and/or standing high-top tables.

# LOADING AREA

Includes use of one bay in our loading dock.

- Dock doors are 8’W x 10’H

- Levelers are available as is a build in scissor lift

- Dock Address: 100 block of North Henry Street, Madison, WI 53703

- Freight elevator: 6’ x 12’ floor; 5’10”W x 7’6”H opening

- Distance to room: 180ft

- Opening to stage/room: 5’6”W x 7’H

# PARKING

Parking for trucks, buses, or production vehicles is available on the street.  Prior arrangements for street parking must be made with the Technical Coordinator; additional charges may apply.

# STAGE DOOR

Actor and crew access is through the main entrance at 201 State Street.

# POWER

- 1 - 200amp, 3-phase, 5-wire disconnect

  - Located in the control room

# BACKSTAGE SPACES

All the support spaces have Wi-Fi internet access or hard-wired internet access available, as well as patchable intercom connections and a television with in-house production CATV as well as local cable channels.

| Room | Location | Quantity | Amenities/Dimensions |
| --- | --- | --- | --- |
| 1-person support space | Stage left | 1 | Includes small shelf and full-length mirror. |
| Technical Booth | Stage right | 1 | Enough space to store some equipment. |

Rotunda Studio is easily accessible from the backstage hallway and is often used as a support space or dressing room. Note that this generally would incur an additional rental fee for the room. Contact the Technical Coordinator for more information.

# STAGE OVERVIEW

STAGE DIMENSIONS

Width: 15’

Depth: 20’

Stage Height: 24”

Height to Ceiling: 10’10”

Height to Lighting Pipes: 10’ 4”

STAGE FLOOR

Sprung maple hardwood floor.  No nailing, lagging, or screwing to the floor is permitted.

SCENIC NOTE

All scenic elements (ex. flats, platforms, stairs, etc) must be back-painted. Overture reserves the right to require proof of flame retardancy for all scenic elements or items coming into the Center, or to “match test” items without verifiable certification.

# HOUSE SOFTGOODS

HOUSE MAIN CURTAIN

Pink Velour, Hung from a custom curved traveler track with center split; curtain opening operated from SR.

# STAGE EQUIPMENT

RISERS **

- Generic 4’ x 6’ platforms with 8”, 12”, 16”, 24” and 32” legs available

- 6 – 3-step choral riser units, 4th step and back railings available

DANCE FLOOR

- Custom cut Harlequin Marley. Black. Roughly 15’W x 20’D total

PIANOS **

- 1 - Steinway 9’ concert grand †

- 2 - Steinway 6’ grand †

- 1 - Yamaha 6’ grand †

- 1 - Yamaha concert upright

- 3 – Rehearsal uprights of various brands

** These items are shared with multiple theatres in the building on a first come first serve basis. Availability cannot be guaranteed and should be advanced with the Director of Production or a Technical Coordinator.

† Only upright pianos can fit on the stage. The grand pianos cannot fit through the ramp entrance to the stage and therefore cannot be placed on the stage. Grands can, however, be placed anywhere in the rear of the room if desired.

# LIGHTING SYSTEM OVERVIEW

STAGE LIGHTING CONTROL SYSTEM

HOUSE/STAGE CONTROL

- House and performance lighting is controlled by a pre-programmed ETC Paradigm System controllable from a touchscreen located around the control room. Various pre-programmed or custom presets can be triggered from these screens, along with direct fixture control.

ADDITIONAL CONSOLE CONTROL

- A portable ETC ION-XE-20 w/ two 24” touchscreens, capable of 24,000+ outputs, 10,000 cues, and 16,000 channels, may be available for more theatrical-type lighting control. Availability is not guaranteed and should be advanced with the Technical Coordinator or Lighting Supervisor. Additional fee may apply.

STAGE LIGHTING CONTROL DISTRIBUTION
The stage utilizes ETC’s Net3 protocol with network access ports located throughout the room. An ETC DMX Gateway is mounted in a rack in the control room. DMX is sent to the lighting positions via DMX-over-CAT5 to accommodate a variety of DMX input and output needs.

HOUSE DIMMING SYSTEM / LIGHTING CIRCUITS

Rotunda Stage has 24 dedicated circuits controlled from an ETC SR48 Sensor rack. Circuit outlet boxes in the ceilings containing 3-5 circuits are dispersed evenly throughout the space.

LIGHTING POSITIONS
(2) Schedule-40 pipes ring the entire space and provide hanging locations for both Overstage and FOH positions. Additionally extra pipe extensions are installed over the stage area.

# LIGHTING INVENTORY

ROTUNDA STAGE DEDICATED INVENTORY

| NAME | # IN REP PLOT | # EXTRA | TOTAL |
| --- | --- | --- | --- |
| ETC ColorSource Spot Jr Zoom | 10 |  | 10 |
| ETC ColorSource PARs | 8 |  | 8 |

DEDICATED INVENTORY PLOT FOCUS

(6) CSSPOTJRs acting as a general front wash, hung (3) each side of center

(6) CSPARs acting as top/side wash over the stage, hung (3) each side of the curtain opening

(2) CSSPOTJRs acting as refocusable specials

(2) CSSPOTJRs acting as pattern wall wash with gobos installed, for walls each side of the stage

(2) CSPARs acting as curtain warmers

ADDITONAL INVENTORY*

| NAME | TOTAL |
| --- | --- |
| High End Systems Halcyon Profile – Titanium, Ultra-Bright † | 8* |
| GLP Impression X5 Wash | 14* |
| ETC Source4 LED – Lustr (Series 1) | 8* |
| ETC Source4 LED – Lustr+ (Series 2) | 20* |
| ETC ColorSource Cycs | 18* |
| ChromaQ Color Force II+ 72” | 16* |
| ETC Source4 MultiPAR (12-cell, 4-circuit) | 12* |
| Altman MR-16 Zip Strips (6-foot, 3-circuit) | 8* |

*These fixtures are shared with multiple theatres in the building. Availability cannot be guaranteed and should be advanced with the Technical Coordinator and/or Lighting Supervisor. Additional rental fees may apply.
† Overture’s Halcyons all have a custom gobo load installed. See document in PDF folder for more information.

MISC LIGHTING EQUIPMENT

- 8 - booms, 12’ tall w/ 50lb. bases, 1 2” I.D. pipe

- 6 – boom trusses, prebuilt with 5 Source Four fixtures each, 7’ tall w/ optional 4’ extension. *

- Numerous pattern holders, top hats, barndoors, drop-in irises, along with sufficient cable, two-fers, quad boxes, etc to hang full inventory.

CEILING RING LIGHTS
The space includes a system of Lumenpulse Lumencove 2.0 (RGBA) architectural fixtures in the two circular ring coves in the ceiling. They are controlled using an ETC Mosaic Show Controller. Users can select between a preset list of color palettes or special patterns from the ETC Paradigm system.

# SOUND SYSTEM OVERVIEW

An installed sound system is included and should cover most needs for this space.  The system includes a full set of L-Acoustics loudspeakers powered and controlled by L-Acoustics amplifiers and network manager. The room features a dedicated mixer, computer for playback, and wireless microphones. Signal may be routed to/from other lobby locations in the building through our Dante-enabled BSS Soundweb network.

FRONT OF HOUSE SOUND CONTROL

- Yamaha QL1 digital mixing console with sixteen built-in microphone inputs.

  - A dedicated Apple Mac Mini with a fully licensed version of QLab 5 installed for playback

  - Apple iPad is included to allow for remote mixing via Yamaha QL StageMix app.

  - Yamaha RIO-3216* and TIO-1608* may be available to allow additional inputs and outputs.

*These are shared throughout the building. Availability should be advanced with the Technical Coordinator.

ASSISTED LISTENING SYSTEM
A Listen Systems FM system is available to assist hearing-impaired patrons.

# HOUSE SPEAKER SYSTEM

SPEAKER LOCATIONS / ZONES

MAIN L/R

- 2 (1 each side) – L-Acoustics A10i speakers

DELAYS

- 4 (2 each side) – L-Acoustics 5XTW speakers

OVERHEAD SIDEFILL MONITORS

- 2 (1 each side) – L-Acoustics X8W speakers

SUBWOOFERS

- 2 (1 each side of the stage) – L-Acoustics SB18

HOUSE AMPLIFIER / SOUND PROCESSING SYSTEM

- L-Acoustics LA2Xi

- L-Acoustics Network Manager

AUDIO INVENTORY*

*Most audio inventory is shared with multiple theatres in the building. Note what is dedicated to the space and what is used building-wide. Availability for building-wide inventory cannot be guaranteed and should be advanced with the Director of Production and/or Technical Coordinator. Additional rental fees may apply.

ROTUNDA STAGE DEDICATED INVENTORY

WIRELESS MICROPHONES

| MODEL NAME | QTY. | # OF CHANNELS | FREQUENCY |
| --- | --- | --- | --- |
| Shure QLXD (QLXD4) | 4 | 4 – QLXD2 Handheld 4 – QLXD1 Bodypack | V50 (174-216 MHz) |

BUILDING-WIDE INVENTORY

WIRELESS MICROPHONES

| MODEL NAME | QTY. | TRANSMITTER OPTIONS | FREQUENCY |
| --- | --- | --- | --- |
| Shure Axient Digital (AD4Q) | 32 | 32 – AD2 Handheld 32 – AD1 Bodypack 4 – ADX1M Micro Bodypack (Microdot) | G57 (470-616 MHz) |
| Shure ULX-D (ULXD4Q) | 16 | 16 – ULXD2 Handheld 16 – ULXD1 Bodypack | G50 (470-534 MHz) |
| Shure UHF-R (UR4D) | 12 | 12 – UR2 Handheld 12 – UR1 Bodypack | 4x G1 (470-530MHz) 8x H4 (518-578MHz) |

All systems include antenna distribution.  All belt packs use TAF4 connectors unless otherwise noted.

CAPSULES FOR WIRELESS

- 2 – Earthworks SR3117

- 2 – Nexadyne 8/C

- 4 – SE Electronics V7 MC1

- 9 – Shure KSM9

- 31 – Shure SM58

- 16 – Shure BETA58

- 20 – Shure BETA58A

- 6 – Shure SM87

- 2 – Shure SM87A

- 2 – Shure SM87C

WIRED MICROPHONES & DI BOXES

- 6 – AKG C414B

- 10 – AKG C451B

- 2 – AKG C480 CK61

- 7 – AKG C535

- 2 – AKG D112

- 1 – Audix D6

- 6 – Audio Technica 4041

- 3 – Audio Technica ATM23

- 3 – Audio Technica ATM25

- 11 – Audio Technica AT8533X

- 3 – Audio Technica Pro35

- 3 – Beyerdynamic M88

- 2 – Beyerdynamic M88 TG

- 2 – Beyerdynamic M201

- 2 – Beyerdynamic M201TG

- 2 – Countryman Type 85 (stereo)

- 12 – Countryman Type 85

- 15 – DPA 4060 (black and beige)

- 19 – DPA 4061 (black and beige)

- 17 – PDA 406 (black and beige)

- 4 – DPA 4099

- 4 – Electro-Voice RE20

AUDIO INVENTORY* (cont’d)

*Audio inventory is shared with multiple theatres in the building. Availability cannot be guaranteed and should be advanced with the Director of Production and/or Sound Supervisor. Additional rental fees may apply.

WIRED MICROPHONES & DI BOXES

- 1 – Electro-Voice PL20

- 7 – Neumann KMS105

- 4 – Neumann KM184

- 2 – Neumann TLM193

- 5 – Neumann TLM170

- 11 – Radial Engineering PRODI

- 4 – SE Electronics V7

- 2 – Sennheiser 441

- 8 – Sennheiser 421

- 4 – Sennheiser e604

- 4 – Sennheiser e904

- 2 – Sennheiser e906

- 12 – Sennheiser MKE2

- 7 – Sennheiser MZH3042

- 4 – Sennheiser MZH3062

- 9 – Shure BETA 52

- 4 – Shure BETA56A

- 8 – Shure BETA 57A

- 9 – Shure BETA 58

- 6 – Shure BETA 87

- 2 – Shure BETA 87C

- 4 – Shure BETA 91

- 4 – Shure KSM137

- 2 – Shure KSM32

- 2 – Shure MX412

- 18 – Shure SM57

- 21 – Shure SM58

- 10 – Shure SM58S

- 4 – Shure SM81

- 10 – VU HM2000

- Numerous – VU LM6000 (black & beige)

- 6 – Whirlwind DIRECTOR Passive DI

- 4 – Whirlwind PCDI

- 4 – Whirlwind PODDI

BACKLINE AMPS

- 1 – Fender Twin Reverb, guitar amplifier, A65 reissue@

- 1 – Gallien-Krueger 400RB, bass guitar amplifier

- 1 – Hartke 115B XL, single 15” bass cabinet

- 1 – Hartke 410 XL, 4x10” bass cabinet

- 1 – Roland JC120 Jazz Chorus keyboard amplifier

STANDS

- STRAIGHT STANDS/ROUND BASE:

  - Atlas   MS-4 (24-56”)

  - Atlas   MS20  (37-66”)

  - Atlas   DS-7 (8-13”)

- BOOM ARMS:

  - Atlas PB77(31”)

  - Atlas   DMS10E (14-26”)

  - Atlas PB21X(25-38”)

  - AKG KM211/1(18-30”)

- STAND/BOOM COMBINATIONS:

  - AKG KM210/91(35-61” stand, 18-30” boom)

  - AKG KM259 (20-28” stand, 18-30” boom)

  - AKG KM259/1 (11” stand, 20” boom)

  - AKG KM259/5 (18” stand, 28.5” boom)

  - AKG KM275 (34-61” stand, 33” boom)

  - AKG KM277 (33.5” stand, 33” boom)

# VIDEO / STREAMING OVERVIEW

*Overture Center has a wide variety of video/streaming assets that can be deployed as needed for productions/events. Most of the equipment is shared throughout the Center (unless otherwise indicated) and therefore availability is not guaranteed. Please inquire with the Director of Production or Video Supervisor for details. Extra rental charges may apply.

VIDEO SERVICES AVAILABLE include:

- Video streaming and recording (up to 16 inputs)

- Hybrid/Virtual meetings (via Zoom or Teams)

- Webinars

- IMAG (Image Magnification)

- Video Playback

- Projection/Surface Mapping

VIDEO/STREAMING INVENTORY*

SCREENS

- 1 – 10’9” x 12’ motorized front projection screen installed on upstage wall.

PROJECTORS

  - 1 – Panasonic PT-MZ16K – 16,000 lm; 4K resolution. (Dedicated to Rotunda Stage, permanently mounted to lighting pipe.)

MONITOR/TV SCREENS

- 10 – 55” HD digital TV screens on rolling carts

- 2 – 65” HD digital TV screens on rolling carts

- 4 – 24” 4K confidence monitors

- 2 – 48” HD digital TV screens with low latency

- 4 – 32” SD analog TVs on rolling carts

CAMERAS

- 4 – Panasonic AW-HE42KPJ PTZ cameras

- 2 – HD PTZ camera (built into room, mounted back of house on lighting pipe.)

VIDEO SWITCHERS

Full production switches come with a cart complete with multiple 4K monitors, Apple Mac Mini, audio mixer, and more. Mini switchers are stand-alone.

- 2 – Blackmagic ATEM 4K HD8 Streaming/Recording Desks

- 1 – Blackmagic ATEM 2M/E Constellation 4K Production Switcher

- 3 – Blackmagic ATEM Mini Pro, HD video switchers

MISCELLANIOUS

- Multiple PC and Apple laptops loaded with all major presentation software

  - Ex. PowerPoint, Keynote, etc

- 4 – DScan Perfect Cue Presentation Remotes

# GROUNDPLAN