PROMENADE HALL TECHNICAL SPECS

CONTENTS

PRODUCTION CONTACTS	3

HOUSE OVERVIEW	3

LOADING AREA	3

PARKING	3

STAGE DOOR	3

BACKSTAGE SPACES	4

STAGE OVERVIEW	4

HOUSE SOFTGOODS	4

STAGE EQUIPMENT	5

POWER	5

LIGHTING SYSTEM OVERVIEW	6

LIGHTING INVENTORY	7

SOUND SYSTEM OVERVIEW	8

HOUSE SPEAKER SYSTEM	9

AUDIO INVENTORY	9

VIDEO / STREAMING OVERVIEW	12

VIDEO/STREAMING INVENTORY	12

MEDIA ROOM	13

GROUNDPLAN – STAGE LEVEL	14

SEATING CHART – FULL RISER + SIDE RISERS	15

# PRODUCTION CONTACTS

Director of Production – John Penisten: 608-258-4451 jpenisten@overture.org

Technical Coordinator – Karen Corbeill: 608-258-4437 kcorbeill@overture.org

Sound Supervisor – Patrick Bley: 608-982-9799 pbley@overture.org

Lighting Supervisor – Zak Stowe: 608-258-4961 zstowe@overture.org

Video Supervisor – Joseph Garcia-Menocal: 608-258-4171 jgarciamenocal@overture.org

# HOUSE OVERVIEW

SEATING BREAKDOWN

Riser Seating:		 			162

Additional Chairs (Thrust config):			90

TOTAL SEATING:			             	250 (flexible)

ADA: Ample seating options available.

MAXIMUM OCCUPANCY: 480

# LOADING AREA

Primary stage access is through the Overture Hall stage door and main loading dock. From loading dock, take the freight elevator to 2nd level to access the backstage area of Promenade Hall.

- Dock Address: 100 block of North Henry Street, Madison, WI 53703

- Dock Available: 1

- Door opening: 8’ W x 10’ H

- Fright Elevator Door: 5’10” W x 7’6” H (6’x12’ Floor Space)

- Opening to stage: 5’6” W x 7’ H

# PARKING

Parking for one truck is available on the street; additional parking for trucks, buses, or production vehicles is available on the street.  Prior arrangements for street parking must be made with the Director of Production; additional charges may apply.

# STAGE DOOR

Actor and crew access is through the main Rotunda entrance or the Overture Hall Stage Door.

# BACKSTAGE SPACES

All the support spaces have Wi-Fi internet access or hard-wired internet access available, as well as patchable intercom connections and a television with in-house production CATV as well as local cable channels.

| Room | Location | Quantity | Amenities/Dimensions |
| --- | --- | --- | --- |
| 3-person Dressing room | Stage left | 1 | 11’ x 16’ – Adjacent to a private bathroom. |

# STAGE OVERVIEW

STAGE DIMENSIONS

Room dimensions:…………………….54’W x 58’ L

Room height:……………………………25’

Grid height:………………………………21’ (bottom of grid)

STAGE FLOOR

Sprung cherry hardwood floor.  No nailing, lagging, or screwing to the floor is permitted.

SCENIC NOTE

All scenic elements (ex. flats, platforms, stairs, etc) must be back-painted. Overture reserves the right to require proof of flame retardancy for all scenic elements or items coming into the Center, or to “match test” items without verifiable certification.

# HOUSE SOFTGOODS

HOUSE SCRIMS

- 1 – black sharkstooth scrim (50’ W x 19’ H)

- 1 – white muslin cyclorama (50’ W x 22’ H)

HOUSE DRAPES –Black velour sewn without fullness; with chain in bottom pocket

- 1 – border (53’ W x 4’ H)

- 3 – legs (6’10” W x 20’ H)

- 6 – legs (10’ W x 20’9” H)

- 1 – legs (8’ W x 20’9” H)

- 2 – panels (21’ W x 20’9” H)

ACOUSTIC CURTAINS

- May be drawn across the upper half of three walls of the room to adjust the room’s acoustic properties.

# STAGE EQUIPMENT

RISERS

- Seating risers pull out from the south wall of the room and accommodate 162 persons.  Fully extended they are 28’ deep and provide nine levels of seating, taking up one-half of the room.

- Generic 3’ x 6’ and 4’ x 6’ platforms with 8”, 12”, 16”, 24” and 32” legs available to create a stage or different seating riser configurations. **

DANCE FLOOR

- 5 – rolls of black/grey (reversable) Super Bravo Marley (6.5’ W x 44’ L)

PIANOS**

- 1 - Steinway 9’ concert grand

- 2 - Steinway 6’ grand

- 1 - Yamaha 6’ grand

- 1 - Yamaha concert upright

- 3 – Rehearsal uprights of various brands

LADDERS/LIFTS

- 2 - Genie GS-1930 Scissor lift **

- 1 – 36’ Genie AWP personnel lift **

- 2 - 8’ fiberglass stepladders

- 2 - 4’ fiberglass stepladders

** These items are shared with multiple theatres in the building on a first come first serve basis. Availability cannot be guaranteed and should be advanced with the Director of Production or a Technical Coordinator.

# POWER

MAIN STAGE POWER

- 1 - 400amp, 3-phase, 5-wire disconnects

  - Located in the backstage hallway

- 1 - 100amp, 3-phase, 5-wire disconnects with earth ground OR isolated ground

  - Located in the backstage hallway

# LIGHTING SYSTEM OVERVIEW

STAGE LIGHTING CONTROL SYSTEM

HOUSE CONSOLE

- ETC ION-XE-20, capable of 24,000+ outputs, 10,000 cues, and 16,000 channels. The console also features 20 manual faders. Accessories include: 2—22” touchscreen monitors.

- ETC Apex Processor, capable of 24K+ outputs that serves as the system’s back-up client.

- Console is on a mobile control desk, along with KMV controls for the QLab and QSys systems, that can be moved from the control booth down to the stage via the freight elevator for extended technical rehearsals.

- Additionally, the EOS system is connected to QLab via OSC for additional control and timecode synchronization.

STAGE LIGHTING CONTROL DISTRIBUTION
The house utilizes ETC’s Net3 protocol with network access ports located throughout the venue including all corners of the stage at stage level and throughout the lighting grid.

Numerous ETC DMX Gateways are available, both installed and portable, to accommodate a variety of DMX input and output needs.

HOUSE LIGHTING CONTROL SYSTEM

House lighting is controlled by a programmable ETC Unison Paradigm system, controllable from the control booth or various locations throughout the room. The Paradigm system can also control stage lighting system. House lights may also be controlled from the lighting console.

HOUSE DIMMERING SYSTEM

- 1 – ETC SR48 Sensor+ dimming rack with various dimming and non-dimming modules providing 96 performance circuits

- 1 – ETC SR24 Sensor+ dimming rack with 14 D20DHR modules providing 24 circuits for house lighting

LIGHTING POSITIONS

- 50’ x 55’ pipe gride with 5’ squares.

LIGHTING CIRCUITS

There are six-circuit wall boxes mounted Stage Left, Stage Right, House Left, and House Right. Wall box circuits are repeated as multicable drops accessible above the wood-laminate wall. There are 6 twelve-circuit connector strips mounted above the pipe grid. Many circuits use dimmer doublers, please see rep plot and dimmer schedule for more info.

# LIGHTING INVENTORY

PROMENADE HALL DEDICATED INVENTORY

| NAME | # IN REP PLOT | # EXTRA | TOTAL |
| --- | --- | --- | --- |
| 19° ETC Source4 lens tube | 4 | 1 | 5 |
| 26° ETC Source4 lens tube | 40 | 3 | 43 |
| 36° ETC Source4 lens tube | 19 | 8 | 27 |
| 50° ETC Source4 lens tube |  | 22 | 22 |
| ETC Source4 Body – 575w | 99 | 30 | 129 |
| ETC Source4 PAR – 575w |  | 6 | 6 |
| ETC Source4 PARnel – 575w | 6 |  | 6 |
| Strand 6” Fresnel – 1kW | 9 | 3 | 12 |
| ETC ColorSource Spot V body | 20* |  | 20 |
| ETC ColorSource PAR – Deep Blue | 12* |  | 12 |
| ETC ColorSource Cyc | 11* |  | 11 |
| Martin Rush Batten 1 HEX |  | 6** | 6 |
| Rosco I-Cue Mirrors, (installed in 2 ColorSource spots, w/ 2 DMX Irises) | 2* |  | 2 |
| ETC Relevé Spots | 2* |  | 2 |

* Additional rental fees may apply.

** These fixtures are owned by Resident companies but are available for use for an additional charge. Availability cannot be guaranteed and should be advanced with the Director of Production and/or Lighting Supervisor.

ADDITONAL INVENTORY*

| NAME | TOTAL |
| --- | --- |
| High End Systems Halcyon Profile – Titanium, Ultra-Bright † | 8* |
| GLP Impression X5 Wash | 14* |
| ETC Source4 LED – Lustr (Series 1) | 8* |
| ETC Source4 LED – Lustr+ (Series 2) | 20* |
| ETC ColorSource Cycs | 18* |
| ChromaQ Color Force II+ 72” | 16* |
| ETC Source4 MultiPAR (12-cell, 4-circuit) | 12* |
| Altman MR-16 Zip Strips (6-foot, 3-circuit) | 8* |
| Reel EFX DF-50 Hazer | 2* |
| Look Solutions Unique 2.1 Hazer | 1* |

*These fixtures are shared with multiple theatres in the building. Availability cannot be guaranteed and should be advanced with the Director of Production and/or Lighting Supervisor. Additional rental fees may apply.

† Overture’s Halcyons all have a custom gobo load installed. See document in PDF folder for more information.

MISC LIGHTING EQUIPMENT

- 8 - booms, 12’ tall w/ 50lb. bases, 1 2” I.D. pipe

- 6 – boom trusses, prebuilt with 5 Source Four fixtures each, 7’ tall w/ optional 4’ extension. *

- Numerous pattern holders, top hats, barndoors, drop-in irises, along with sufficient cable, two-fers, quad boxes, etc to hang full inventory.

PARTY LIGHTS
14 Blizzard HoxBox EXA fixtures are installed around the room as accent downlight grazing the walls. Fixtures can be controlled from Paradigm or lighting console. Additionally, 12 pairs of HotBoxes are installed as downlights throughout the space.

# SOUND SYSTEM OVERVIEW

Promenade Hall’s sound system consists of a left and right array with subs, fill speakers for side seating and rear delay fill speakers for better coverage of the top row of the bleacher seats.  The system is controlled via Q-Sys control software and includes 14 mic/line level system inputs, a built-in wireless mic (handheld or lavalier, an iPod input and 3 AV (line level) inputs.

FRONT OF HOUSE MIXING CONSOLE

- QSC QSys System serves as the main room sound control. The main control desk features 2 KVM set-ups to allow remote control of Q-Sys computer controls and QLab Mac. Desk can be moved down from the control booth to the stage for extended technical rehearsals. If greater sound mixing capabilities are needed the following might be available for use in the space:

Depending on availability the following may be available:

- YAMAHA CL5, with 1 – MY-AUD2 card, 1 – MY8-ADDA96 card, and 1 – MY8-AE96S card.

  - Included with the console is a 2020 MacMini (M1 CPU 16GB RAM) running Yamaha VST Rack Pro software interfaced via Dante with a Yamaha RIO-1-D.

  - Includes all factory plug-ins along with Waves Create Access Ultimate.

  - Stage Connections are via (2) RIO-3224 digital stage boxes for a total of 64 mic/line inputs, 32 analog line outputs, and 16 AES/EBU outputs.

- YAMAHA CL3 and CL1 with similar accessories may also be available. Inquire with Director of Production.

PLAYBACK SYSTEMS

- 1 - QLab 5 – Full audio and video license for playback via Apple Mac Mini (M1 CPU 16GB RAM) with Dante Virtual Soundcard to interface with Q-Sys system. Additionally, QLab is connected to the EOS lighting network via OSC for additional control and timecode synchronization.

PROGRAM & PAGING SYSTEMS

Each input in QSys can be assigned to “Program” which feeds the backstage dressing rooms, audience hearing assist, in house TV system and program XLR connections around the venue.  In addition, mics on the balcony rail provide ambient sound to feed the program system as well.

ASSISTED LISTENING SYSTEM
A Listen Technologies FM hearing assist system is used to provide ADA required hearing assist.  Patrons can pick up these headsets at the info desk.  Only a mute and level control are available for adjustment.  This system should be tested before each show to ensure it is properly working. Test units are in the sound booth.  It is a required system for every event.

INTERCOM SYSTEM

Intercom is a Clear-Com system with a 2-channel power supply.  Several single and dual channel belt packs and headsets are available.  Intercom connection locations are distributed throughout the theater space.

# HOUSE SPEAKER SYSTEM

SPEAKER LOCATIONS / ZONES

MAIN ARRAYS

- 12 (6 each side) – Nexo Geo M620 Speakers

- 2 (1 each side) – Nexo LS600 Subwoofers

SIDE FILLS

- 4 (2 each side) – Nexo ID24 (90x40) speakers used for side seating when applicable

REAR FILLS

- 4 – Nexo ID24 (90x40) speakers used to cover top row of riser seating

HOUSE AMPLIFIER SYSTEM

- 3 – Nexo NXAMP4X1 amplifiers for mains and fills

- 4 – QSC RMX2450 amplifiers for 8 channels of effects sends.

AUDIO INVENTORY*

*Most audio inventory is shared with multiple theatres in the building. Note what is dedicated to the space and what is used building-wide. Availability for building-wide inventory cannot be guaranteed and should be advanced with the Director of Production and/or Technical Coordinator. Additional rental fees may apply.

PROMENADE HALL DEDICATED INVENTORY

WIRELESS MICROPHONES

| MODEL NAME | # OF CHANNELS | FREQUENCY |
| --- | --- | --- |
| Shure QLXD (QLXD4) | 1 channel, both handheld and belt pack transmitters. | V50 (174-216 MHz) |

BUILDING-WIDE INVENTORY

WIRELESS MICROPHONES

| MODEL NAME | # OF CHANNELS | FREQUENCY |
| --- | --- | --- |
| Shure AD1 Axient (AD4Q) | 32 channels, both handheld and belt pack transmitters. | G57 (470-616 MHz) |
| Shure ULXD (ULXD4Q) | 16 channels, both handheld and belt pack transmitters. | G50 (470-534 MHz) |
| Shure UHF-R (UR4D) | 12 channels, both handheld and belt pack transmitters. | 4x G1 (470-530MHz) 8x H4 (518-578MHz) |

All systems include antenna distribution.  All belt packs use TAF4 connectors.

AUDIO INVENTORY* (cont’d)

*Most audio inventory is shared with multiple theatres in the building. Note what is dedicated to the space and what is used building-wide. Availability for building-wide inventory cannot be guaranteed and should be advanced with the Director of Production and/or Technical Coordinator. Additional rental fees may apply.

CAPSULES FOR WIRELESS

- 2 – Earthworks SR3117

- 2 – Nexadyne 8/C

- 4 – SE Electronics V7 MC1

- 9 – Shure KSM9

- 31 – Shure SM58

- 16 – Shure BETA58

- 20 – Shure BETA58A

- 6 – Shure SM87

- 2 – Shure SM87A

- 2 – Shure SM87C

WIRED MICROPHONES & DI BOXES

- 6 – AKG C414B

- 10 – AKG C451B

- 2 – AKG C480 CK61

- 7 – AKG C535

- 2 – AKG D112

- 1 – Audix D6

- 6 – Audio Technica 4041

- 3 – Audio Technica ATM23

- 3 – Audio Technica ATM25

- 11 – Audio Technica AT8533X

- 3 – Audio Technica Pro35

- 3 – Beyerdynamic M88

- 2 – Beyerdynamic M88 TG

- 2 – Beyerdynamic M201

- 2 – Beyerdynamic M201TG

- 2 – Countryman Type 85 (stereo)

- 12 – Countryman Type 85

- 15 – DPA 4060 (black and beige)

- 19 – DPA 4061 (black and beige)

- 17 – PDA 406 (black and beige)

- 4 – DPA 4099

- 4 – Electro-Voice RE20

- 1 – Electro-Voice PL20

- 7 – Neumann KMS105

- 4 – Neumann KM184

- 2 – Neumann TLM193

- 5 – Neumann TLM170

- 11 – Radial Engineering PRODI

- 4 – SE Electronics V7

- 2 – Sennheiser 441

- 8 – Sennheiser 421

- 4 – Sennheiser e604

- 4 – Sennheiser e904

- 2 – Sennheiser e906

- 12 – Sennheiser MKE2

- 7 – Sennheiser MZH3042

- 4 – Sennheiser MZH3062

- 9 – Shure BETA 52

- 4 – Shure BETA56A

- 8 – Shure BETA 57A

- 9 – Shure BETA 58

- 6 – Shure BETA 87

- 2 – Shure BETA 87C

- 4 – Shure BETA 91

- 4 – Shure KSM137

- 2 – Shure KSM32

- 2 – Shure MX412

- 18 – Shure SM57

- 21 – Shure SM58

- 10 – Shure SM58S

- 4 – Shure SM81

- 10 – VU HM2000

- Numerous – VU LM6000 (black & beige)

- 6 – Whirlwind DIRECTOR Passive DI

- 4 – Whirlwind PCDI

- 4 – Whirlwind POD

AUDIO INVENTORY* (cont’d)

*Most audio inventory is shared with multiple theatres in the building. Note what is dedicated to the space and what is used building-wide. Availability for building-wide inventory cannot be guaranteed and should be advanced with the Director of Production and/or Technical Coordinator. Additional rental fees may apply.

BACKLINE AMPS

- 1 – Fender Twin Reverb, guitar amplifier, A65 reissue@

- 1 – Gallien-Krueger 400RB, bass guitar amplifier

- 1 – Hartke 115B XL, single 15” bass cabinet

- 1 – Hartke 410 XL, 4x10” bass cabinet

- 1 – Roland JC120 Jazz Chorus keyboard amplifier

STANDS

- STRAIGHT STANDS/ROUND BASE:

  - Atlas MS-4 (24-56”)

  - Atlas MS20  (37-66”)

  - Atlas DS-7 (8-13”)

- BOOM ARMS:

  - Atlas PB77(31”)

  - Atlas DMS10E (14-26”)

  - Atlas PB21X(25-38”)

  - K&M KM211/1(18-30”)

- STAND/BOOM COMBINATIONS:

  - K&M KM210/91(35-61” stand, 18-30” boom)

  - K&M KM259 (20-28” stand, 18-30” boom)

  - K&M KM259/1 (11” stand, 20” boom)

  - K&M KM259/5 (18” stand, 28.5” boom)

  - K&M KM275 (34-61” stand, 33” boom)

  - K&M KM277 (33.5” stand, 33” boom)

# VIDEO / STREAMING OVERVIEW

VIDEO SERVICES AVAILABLE include:

- Video streaming and recording (<16 inputs)

- Hybrid/Virtual meetings (via Zoom or Teams)

- Webinars

- IMAG (Image Magnification)

- Video Playback

- Projection/Surface Mapping

VIDEO/STREAMING INVENTORY*

*Most video inventory is shared with multiple theatres in the building. Note what is dedicated to the space and what is used building-wide. Availability for building-wide inventory cannot be guaranteed and should be advanced with the Director of Production and/or Technical Coordinator. Additional rental fees may apply.

SCREENS

- 1 – 9’H x 16’W motorized front projection screen (built into Promenade Hall)

PROJECTORS

  - 1 – Panasonic PT-MZ16K – 16,000 lm; 4K resolution. (built into Promenade Hall)

    - While its primary purpose is to project on the installed screen, a complement of lenses is available to accommodate use as a scenic projector for theatrical production.

  - 2 – Panasonic PT-MX16KBU77 – 16,000 lm; 4K resolution

MONITOR/TV SCREENS

- 10 – 55” HD digital TV screens on rolling carts

- 2 – 65” HD digital TV screens on rolling carts

- 4 – 24” 4K confidence monitors

- 2 – 48” HD digital TV screens with low latency

- 4 – 32” SD analog TVs on rolling carts

CAMERAS

- 4 – Panasonic AW-HE42KPJ PTZ cameras

- 1 – HD PTZ camera (built into Promenade Hall)

- 3 – Sony DXC-390 3CCD Cameras w/ Canon YH19x6.7 KRS lens (as conductor cams)

VIDEO SWITCHERS

Full production switches come with a cart complete with multiple 4K monitors, Apple Mac Mini, audio mixer, and more. Mini switchers are stand-alone.

- 2 – Blackmagic ATEM 4K HD8 Streaming/Recording Desks

- 1 – Blackmagic ATEM 2M/E Constellation 4K Production Switcher

- 3 – Blackmagic ATEM Mini, HD video switchers

MISCELLANIOUS

- Multiple PC and Apple laptops loaded with all major presentation software

  - Ex. PowerPoint, Keynote, etc

- 4 – DScan Perfect Cue Presentation Remotes

# MEDIA ROOM

Overture Center is equipped with a media connection room, accessible via a cable pass-through to Henry Street.  This room provides Audio (XLR) and Video (BNC) connections to nine separate locations throughout the building.

- 2 – locations in the Overture Hall Lobby

- 3 – locations in Overture Hall (Broadcast booth, Projection booth and H.R. Mezzanine)

- 1 – located in the Capitol Theater (Sound booth)

- 1 – located in The Playhouse (House Left slit)

- 1 – located in Promenade Hall

- 1 – located in MMoCA Lecture Hall

Each location has three groups of connections.  Each group has two male and two female XLR audio connections, and two female BNC composite video connections.

# GROUNDPLAN – STAGE LEVEL

# SEATING CHART – FULL RISER + SIDE RISERS