PLAYHOUSE TECHNICAL SPECS

CONTENTS

PRODUCTION CONTACTS	3

HOUSE OVERVIEW	3

LOADING AREA	3

PARKING	3

STAGE DOOR	3

BACKSTAGE SPACES	4

STAGE OVERVIEW	4

FLY/RIGGING SYSTEM	5

HOUSE SOFTGOODS	5

STAGE EQUIPMENT	6

LIGHTING SYSTEM OVERVIEW	7

HOUSE LIGHTING POSITIONS	8

POWER	8

LIGHTING INVENTORY	9

SOUND SYSTEM OVERVIEW	10

HOUSE SPEAKER SYSTEM	11

AUDIO INVENTORY	13

VIDEO / STREAMING OVERVIEW	15

VIDEO/STREAMING INVENTORY	15

MEDIA ROOM	16

GROUNDPLAN – STAGE LEVEL	17

GROUNDPLAN – RIGGING/LIGHTING	18

GROUNDPLAN – SECTION VIEW	19

PLAYHOUSE SEATING CHARTS	20

# PRODUCTION CONTACTS

Director of Production – John Penisten: 608-258-4451 jpenisten@overture.org

Technical Coordinator – Karen Corbeill: 608-258-4437 kcorbeill@overture.org

Sound Supervisor – Patrick Bley: 608-982-9799 pbley@overture.org

Lighting Supervisor – Zak Stowe: 608-258-4961 zstowe@overture.org

Video Supervisor – Joseph Garcia-Menocal: 608-258-4171 jgarciamenocal@overture.org

# HOUSE OVERVIEW

SEATING BREAKDOWN

Orchestra (permanent seats):   			226

Orchestra (chairs):				20

Balcony (permanent seats): 			81

Balcony (chairs):					20

TOTAL SEATING:			             	347

ADA: Ample seating options available.

# LOADING AREA

Primary stage access is through a roll-up door at street level. From door, access is down a ramped hallway to the backstage area.

- Dock Address: 100 block of North Henry Street, Madison, WI 53703

- Door opening: 10’ W x 10’2” H

- Opening to stage: 7’10” W x 9’ 10” H

- Distance to stage: 100’

# PARKING

Parking for one truck is available on the street; additional parking for trucks, buses, or production vehicles is available on the street.  Prior arrangements for street parking must be made with the Director of Production; additional charges may apply.

# STAGE DOOR

Actor and crew access is through a white rolling garage door, with build-in pedestrian door on North Henry Street.

# BACKSTAGE SPACES

All the support spaces have Wi-Fi internet access or hard-wired internet access available, as well as patchable intercom connections and a television with in-house production CATV as well as local cable channels.

| Room | Location | Quantity | Amenities/Dimensions |
| --- | --- | --- | --- |
| 4-6 person dressing rooms | Stage right | 3 | Showers and toilet |
| 10-12 person dressing room | Under house | 1 | Showers and toilets |
| Green Room | Stage left | 1 | 20’ X 13’ with refrigerator, sink & microwave |
| Wardrobe Workroom | Stage right | 1 | 32’ X 12’ with washer/dryer and sink |

# STAGE OVERVIEW

STAGE DIMENSIONS

Proscenium width: …………………….36’

Proscenium height:……………………15’-21’ adjustable

Stage width:………………………………28’0” (from nearest object stage right to center line)

30’6” (from nearest object stage left to center line)

Stage depth: …………………………….16’0” (from proscenium to first obstruction)

Apron depth:…………………………… 36” (permanent extension from proscenium into house)

Stage height:……………………………..21”

THRUST

- Constructed with 3’x6’ and 3’x3’ platforms with a ¾” Plyron deck.

- Minimum size of 18’ wide x 15’ deep, maximum size of 24’ wide x 21’ deep.

- Thrust platforms may be removed to reveal a secondary set of Plyron-decked platforms that make up the floor. These platforms may also be removed to reveal the trap room below.

- The thrust begins 33’ downstage of the proscenium line.

STAGE FLOOR

Upstage of proscenium is 1/8” Masonite top surface, bonded to 5/8” plywood. The forestage thrust is ¾” Plyron decked platforms which can be removed or rearranged as needed. The floor below the thrust is made up of ¾” Plyron decked platforms. These may also be removed to access trap space below.

SCENIC NOTE

All scenic elements (ex. flats, platforms, stairs, etc) must be back-painted. Overture reserves the right to require proof of flame retardancy for all scenic elements or items coming into the Center, or to “match test” items without verifiable certification.

# FLY/RIGGING SYSTEM

Lock Rail: …………………………………Stage right at stage level

Load Gallery: ……………………………Stage right, 20’ above stage floor

Pin Rail:	…………………………………….Stage right on load gallery

Counterweight System: …….………..8—ETC Prodigy Balance counterweight hoists w/ ETC Foundation control

Counterweight: ………………………….22,000 lbs. available

Batten material: ………………………….1 ½” nom. Schedule 40 pipe

Batten length: …………………………….Varies, see ground plan/lighting plot

Hemp System: …………………………...30-single, 6-double, 6-triple, and 3 -under hung, triple line mule blocks 
 				  2000' of 5/8" Multiline II, sandbags, pipe, etc.

NOTES

- 1-dead hung traveler at 5’ 2”’ upstage of the proscenium line.

- 1-dead hung pipe at 12’ that is at a height of 22’ typically used as a cyc electric

- 1-dead hung pipe at 9’ 6” that is at a height of 22’

- 2-multiline system pipes at 11’ and 16’ upstage of the proscenium line, run from the upstage center pinrail typically used for house black scrim and house cyc, respectively.

- Apron grid runs on ETC Prodigy hoist, controlled from ETC Foundation touchscreen at stage left.

# HOUSE SOFTGOODS

HOUSE SCRIMS

- 1 – black sharkstooth scrim (42’W x 19’H)

- 1 – white leno filled scrim (50’W x 22’H)

- 1 – translucent drop (50’W x 21’9”H)

HOUSE DRAPES –Black velour sewn without fullness; with chain in bottom pocket

- 1 – border (42’ w x 4’ h)

- 1 – border (42’ w x 6’ h)

- 12 – legs (8’ w x 22’ h)

- 2 – panels (32’ w x 22’ h)

# STAGE EQUIPMENT

RISERS

- Generic 4’ x 6’ platforms with 8”, 12”, 16”, 24” and 32” legs available

- 6 – 3-step choral riser units, 4th step and back railings available

DANCE FLOOR

- Custom-cut Harlequin Marley to fit stage and thrust, black, roughly 24’W x 26’8”D

PIANOS

- 1 - Steinway 9’ concert grand

- 2 - Steinway 6’ grand

- 1 - Yamaha 6’ grand

- 1 - Yamaha concert upright

- 3 – Rehearsal uprights of various brands

LADDERS/LIFTS

- 1 - 24’ Genie AWP personnel lift

- 1 - Genie GS-1930 Scissor lift (availability not guaranteed, confirm with Director of Production)

- 1 – Werner MT17 Multiposition ladder

- 2 - 8’ fiberglass stepladders

- 2 - 4’ fiberglass stepladders

# LIGHTING SYSTEM OVERVIEW

STAGE LIGHTING CONTROL SYSTEM

HOUSE CONSOLE

- ETC Apex 5, capable of 24,000+ outputs, 10,000 cues, and 16,000 channels. The console also features 5 motorized playback faders and a 24” 4K touchscreen. Accessories include: 2 additional 22” touchscreen monitors and an ETC 20-fader Standard Fader wing.

- ETC Apex Processor, capable of 24K+ outputs that serves as the system’s back-up client.

ADDITIONAL CLIENT CONSOLES

- ETC Nomad (via PC laptop) w/ one 24” touchscreen and an ETC Programming Wing for “tech table” set-ups.

STAGE LIGHTING CONTROL DISTRIBUTION
The house utilizes ETC’s Net3 protocol with network access ports located throughout the venue including all corners of the stage at stage level, balcony rail, loading gallery, all catwalks over the house, booth, and trap room.

Numerous ETC DMX Gateways are available, both installed and portable, to accommodate a variety of DMX input and output needs.

HOUSE LIGHTING CONTROL SYSTEM

House lighting is operated by a pre-programmed ETC Paradigm system controllable from stage left, house right entrance, or the lighting control booth.  Several house light presets are available to meet your needs.  House light control through the lighting console is not possible.

HOUSE DIMMERING SYSTEM

- 3 – ETC SR48 Sensor4 dimming racks with various dimming and non-dimming modules

  - 276 – 20A circuits

  - 6 – 50A circuits

- 1 – ETC SR12 Sensor portable tack located in the upstage hallway. This rack powers dead hung cyc electric and provides an additional 2- circuits in portable 4- circuit drop boxes with at least 100’ of cable each.

# HOUSE LIGHTING POSITIONS

| NAME | LOCATION | # CIRCUITS |
| --- | --- | --- |
| CATWALKS | (3) Catwalks extend fully left and right over forestage, and L/R along sidewalls, 2’ above stage | 136 – 20A circuits distributed throughout catwalks |
| BALCONY RAIL | Combination of pipe downstage, and Unistrut stage L/R, 19’ to stage lip | 21 – 20A circuits |
| FRONT OF HOUSE | 40’ to stage lip, high and low pipes | 12 – 20A circuits, some of which are sent upstage to proscenium |
| BACKSTAGE | 10 – 6-circuit multi connections to power batons. | 60 – 20A circuits |
| MISC | Circuits located in trap room, house, and booth |  |

# POWER

MAIN STAGE POWER

- 1 - 400amp, 3-phase, 5-wire disconnects

  - Far Upstage Left, used for portable dimmer rack

- 3 - 100amp, 3-phase, 5-wire disconnects

  - Upstage right

  - Stage Right at Loading Gallery

  - Trap Room DSR

- 1 - 60amp, 3-phase, 4-wire disconnects

  - House right, back of house at catwalk level

# LIGHTING INVENTORY

PLAYHOUSE DEDICATED INVENTORY

| NAME | # IN REP PLOT | # EXTRA | TOTAL |
| --- | --- | --- | --- |
| 10° ETC Source4 lens tube |  | 2 | 2 |
| 19° ETC Source4 lens tube | 10 | 34 | 44 |
| 26° ETC Source4 lens tube | 30 | 55 | 85 |
| 36° ETC Source4 lens tube | 18 | 29 | 47 |
| 50° ETC Source4 lens tube |  | 14 | 14 |
| ETC Source4 body – 575w | 22 | 168 | 190 |
| ETC Source4 PAR – 575w |  | 14 | 14 |
| ETC Source4 PARnel – 575w |  | 10 | 10 |
| Altman PAR64, various 1000w lamps available |  | 68 | 68 |
| Strand 6” Fresnel – 1kW |  | 24 | 24 |
| ETC ColorSource Spot V body | 36 |  | 36 |
| ETC ColorSource PAR V Zoom | 12 | 6* | 18 |
| ETC ColorSource PAR |  | 5* | 5 |
| ETC ColorSource Cyc | 7* |  | 7 |
| High End Systems Lonestar | 8* |  | 8 |

* Additional rental fees may apply. Must be advanced with the Director of Production and/or Lighting Supervisor.

ADDITONAL INVENTORY*

| NAME | TOTAL |
| --- | --- |
| High End Systems Halcyon Profile – Titanium, Ultra-Bright † | 8* |
| GLP Impression X5 Wash | 14* |
| ETC Source4 LED – Lustr (Series 1) | 8* |
| ETC Source4 LED – Lustr+ (Series 2) | 20* |
| ETC ColorSource Cycs | 18* |
| ChromaQ Color Force II+ 72” | 16* |
| ETC Source4 MultiPAR (12-cell, 4-circuit) | 12* |
| Altman MR-16 Zip Strips (6-foot, 3-circuit) | 8* |
| Reel EFX DF-50 Hazer | 2* |
| Look Solutions Unique 2.1 Hazer | 1* |

*These fixtures are shared with multiple theatres in the building. Availability cannot be guaranteed and should be advanced with the Director of Production and/or Lighting Supervisor. Additional rental fees may apply.
† Overture’s Halcyons all have a custom gobo load installed. See document in PDF folder for more information.

MISC LIGHTING EQUIPMENT

- 10 - booms, 16’ tall w/ 50lb. bases, 1 2” I.D. pipe

- 6 – boom trusses, prebuilt with 5 Source Four fixtures each, 7’ tall w/ optional 4’ extension.

- Numerous pattern holders, top hats, barndoors, drop-in irises, along with sufficient cable, two-fers, quad boxes, etc to hang full inventory.

# SOUND SYSTEM OVERVIEW

The Playhouse’s sound system consist of a center array, left / right arrays, surround speakers, effects subwoofer, center overhead monitor and individual monitor and effects speakers.  Amplification is built into the system and control of most aspects of the sound is done through Soundweb London control software or installed installed Lake I/O with Dante network.  Additionally, an Apple computer operates the Qlab application for sound and/or video effects playback

FRONT OF HOUSE MIXING CONSOLE

- YAMAHA CL5, with 1 – MY-AUD2 card, 1 – MY8-ADDA96 card, and 1 – MY8-AE96S card.

  - Included with the console is a 2020 MacMini (M1 CPU 16GB RAM) running Yamaha VST Rack Pro software interfaced via Dante with a Yamaha RIO-1-D.

  - Includes all factory plug-ins along with Waves Create Access Ultimate.

  - Stage Connections are via (2) RIO-3224 digital stage boxes for a total of 64 mic/line inputs, 32 analog line outputs, and 16 AES/EBU outputs.

Depending on availability the following may be available:

- YAMAHA CL3 and CL1 with similar accessories may also be available. Inquire with Director of Production.

PLAYBACK SYSTEMS

- 1 - Qlab 5 - audio playback via Apple Mac Mini (M1 CPU 16GB RAM)

PROGRAM & PAGING SYSTEMS

Each input in Soundweb can be assigned to “Program” which feeds the backstage dressing rooms, audience hearing assist, in house TV system and program XLR connections around the venue.  In addition, mics on the balcony rail provide ambient sound to feed the program system as well.

ASSISTED LISTENING SYSTEM
An RF hearing assist system is used to provide ADA required hearing assist.  Patrons can pick up these headsets at the info desk.  Only a mute and level control are available for adjustment.  This system should be tested before each show to ensure it is properly working. Test units are in the sound booth.  It is a required system for every event.

INTERCOM SYSTEMS

Intercom is a Clear-Com system with a 2-channel power supply.  Several single and dual channel belt packs and headsets are available.  Intercom connection locations are distributed throughout the theater space.

WIRELESS INTERCOM
Clear-Com system with 2 channels includes 10 hardwired belt packs. ClearCom FreeSpeak is also installed with 4 wireless belt packs available.

# HOUSE SPEAKER SYSTEM

SPEAKER LOCATIONS / ZONES

CENTER ARRAY
Located on center, above the thrust stage, just under the second catwalk and provides complete and even coverage to all seats from a central monaural source.  The array consists of the following components:

- 6 – JBL AC2895 / AC2826 loudspeakers each with dual 8” low frequency and 1” high frequency loudspeakers, permanently hung under the middle catwalk.  Hung 2 high, 3 wide providing coverage for the main floor left, center and right as well as the balcony left, center and right.

- 2 – JBL ASB6112 single 12” low frequency subwoofers hung above the catwalk compliment the system for full range coverage but can be muted for applications which require a bandwidth limited response.  The system crosses over at 80Hz.

- 4 – JBL AC25 dual 5.25” speakers hung under the balcony and wired in pairs (left and right) to provide complete coverage for the last row of main floor under balcony seating (stools). These are time and frequency aligned to the center main speaker.

Each input in Soundweb, or Lake I/O with Dante, allows the signal to be routed and EQ-ed to the entire center center array (standard) or, if desired, to the individual paired array speakers (left, right, center) depending on the designers needs.  Under balcony speakers can be assigned directly in pairs (right pair, left pair) or, if the complete array is assigned, a portion of the center main floor speaker is routed to the under-balcony speakers.  The “Outputs” page in Soundweb allows individual control of master array level, individual array speaker levels, individual under balcony or “center array lows” (subs) as well as mutes for all options.

LEFT AND RIGHT ARRAYS
Hung on the Proscenium wall on both main floor and balcony levels, they provide a wide coverage pattern for most seats as well as the thrust portion of the stage.  These speakers offer a nice stereo presentation but any live mics on the thrust portion of the stage would be in front of the coverage pattern making feedback a concern.  They are very good for amplifying music or an orchestra located upstage.  The Left / Right arrays consist of the following:

- 4 JBL CWT128 dual 8” dual 1” loudspeakers hung Left & right main floor and left & right balcony.

Each input in Soundweb, or Lake I/O with Dante, allows the signal to be routed and EQ-ed either to the entire left array, right array, or to individual speakers depending on designer’s needs.  The “Outputs” page in Soundweb, or in Lake I/O, allows individual control of master array level, individual array speaker levels, or “left / right array lows” (subs) and mutes for all options.

SURROUND SYSTEM
A series of 8 speakers on main floor level and 8 speakers on balcony level are distributed throughout the audience area to provide even coverage of surround sound to all seats. Each speaker is connected to individual amplifier channels providing a total of 16 possible surround sources.  Each input in Soundweb, or Lake I/O, can be assigned to any of the 16 total surround speakers.  Internal high pass filters block extreme low frequencies to help protect the small surround speakers from attempting to reproduce frequencies beyond their abilities.  Each speaker can provide direct sound to the audience or have its mounting bracket rotated to bounce the sound off the wall for a more diffused sound depending on the designer’s needs.  The surround system is comprised of the following:

- 8 JBL AC15 single 5.25” two-way loudspeaker.

Other speaker options may be plugged into the Speak-on connections and located in other locations but the stock speakers should never be removed or dismantled.

EFFECTS SPEAKERS
Soundweb and Lake I/O allows each input to be routed to any of the eight “Effects” speaker outputs.  Effects speakers can be any of the “floating” utility speakers or most other full range or amplified speakers.  The amplifier output Speak-on connections are terminated on the sound booth rack front and can be patched to any of the locations around the venue.  Additionally, line level XLR outputs for each of the effects speaker feeds are also located on the sound booth rack and can be patched to any needed location for self-powered speakers.  Standard included effects speaker options include:

- 4 JBL AC16, 6-inch two-way loudspeaker with hanging brackets

- 4 JBL SRX812, 12-inch two-way wedge monitor loudspeakers

- Other options available on request

EFFECTS SUBWOOFERS
Hung above and between the 2nd and 3rd catwalk, the Effect Subwoofer is a separate effect which can be used in conjunction or independently with any of the system.  Its design is primarily as an effect, it can shake the room when required.  Consists of:

- 1 JBL ASB7128, 2 x 18” subwoofer

MONITORS

A center hung monitor speaker is installed just behind the center array speakers pointing at the thrust portion of the stage. Signal can be assigned from any of the input sources.  The monitor speaker consists of:

- 1 JBL AC 16 single 6” two-way loudspeaker.

HOUSE AMPLIFIER SYSTEM

- 5 – Lab Gruppen D 80:4L, Dante enabled

- 6 – Lab Gruppen D 40:4L, Dante enabled

AUDIO INVENTORY*
*Most audio inventory is shared with multiple theatres in the building. Note what is dedicated to the space and what is used building-wide. Availability for building-wide inventory cannot be guaranteed and should be advanced with the Director of Production and/or Technical Coordinator. Additional rental fees may apply.

PLAYHOUSE DEDICATED INVENTORY

WIRELESS MICROPHONES

| MODEL NAME | QTY. | # OF CHANNELS | FREQUENCY |
| --- | --- | --- | --- |
| Shure QLXD (QLXD4) | 16 | 16 – QLXD2 Handheld 16 – QLXD1 Bodypack | V50 (174-216 MHz) |

BUILDING-WIDE INVENTORY

WIRELESS MICROPHONES

| MODEL NAME | QTY. | TRANSMITTER OPTIONS | FREQUENCY |
| --- | --- | --- | --- |
| Shure Axient Digital (AD4Q) | 32 | 32 – AD2 Handheld 32 – AD1 Bodypack 4 – ADX1M Micro Bodypack (Microdot) | G57 (470-616 MHz) |
| Shure ULX-D (ULXD4Q) | 16 | 16 – ULXD2 Handheld 16 – ULXD1 Bodypack | G50 (470-534 MHz) |
| Shure UHF-R (UR4D) | 12 | 12 – UR2 Handheld 12 – UR1 Bodypack | 4x G1 (470-530MHz) 8x H4 (518-578MHz) |

All systems include antenna distribution.  All belt packs use TAF4 connectors unless otherwise noted.

CAPSULES FOR WIRELESS

- 2 – Earthworks SR3117

- 2 – Nexadyne 8/C

- 4 – SE Electronics V7 MC1

- 9 – Shure KSM9

- 31 – Shure SM58

- 16 – Shure BETA58

- 20 – Shure BETA58A

- 6 – Shure SM87

- 2 – Shure SM87A

- 2 – Shure SM87C

WIRED MICROPHONES & DI BOXES

- 6 – AKG C414B

- 10 – AKG C451B

- 2 – AKG C480 CK61

- 7 – AKG C535

- 2 – AKG D112

- 1 – Audix D6

- 6 – Audio Technica 4041

- 3 – Audio Technica ATM23

- 3 – Audio Technica ATM25

- 11 – Audio Technica AT8533X

- 3 – Audio Technica Pro35

- 3 – Beyerdynamic M88

- 2 – Beyerdynamic M88 TG

- 2 – Beyerdynamic M201

- 2 – Beyerdynamic M201TG

- 2 – Countryman Type 85 (stereo)

- 12 – Countryman Type 85

- 15 – DPA 4060 (black and beige)

- 19 – DPA 4061 (black and beige)

- 17 – PDA 406 (black and beige)

- 4 – DPA 4099

- 4 – Electro-Voice RE20

AUDIO INVENTORY* (cont’d)

*Most audio inventory is shared with multiple theatres in the building. Note what is dedicated to the space and what is used building-wide. Availability for building-wide inventory cannot be guaranteed and should be advanced with the Director of Production and/or Technical Coordinator. Additional rental fees may apply.

WIRED MICROPHONES & DI BOXES (cont’d)

- 1 – Electro-Voice PL20

- 7 – Neumann KMS105

- 4 – Neumann KM184

- 2 – Neumann TLM193

- 5 – Neumann TLM170

- 11 – Radial Engineering PRODI

- 4 – SE Electronics V7

- 2 – Sennheiser 441

- 8 – Sennheiser 421

- 4 – Sennheiser e604

- 4 – Sennheiser e904

- 2 – Sennheiser e906

- 12 – Sennheiser MKE2

- 7 – Sennheiser MZH3042

- 4 – Sennheiser MZH3062

- 9 – Shure BETA 52

- 4 – Shure BETA56A

- 8 – Shure BETA 57A

- 9 – Shure BETA 58

- 6 – Shure BETA 87

- 2 – Shure BETA 87C

- 4 – Shure BETA 91

- 4 – Shure KSM137

- 2 – Shure KSM32

- 2 – Shure MX412

- 18 – Shure SM57

- 21 – Shure SM58

- 10 – Shure SM58S

- 4 – Shure SM81

- 10 – VU HM2000

- Numerous – VU LM6000 (black & beige)

- 6 – Whirlwind DIRECTOR Passive DI

- 4 – Whirlwind PCDI

- 4 – Whirlwind POD

BACKLINE AMPS

- 1 – Fender Twin Reverb, guitar amplifier, A65 reissue@

- 1 – Gallien-Krueger 400RB, bass guitar amplifier

- 1 – Hartke 115B XL, single 15” bass cabinet

- 1 – Hartke 410 XL, 4x10” bass cabinet

- 1 – Roland JC120 Jazz Chorus keyboard amplifier

STANDS

- STRAIGHT STANDS/ROUND BASE:

  - Atlas MS-4 (24-56”)

  - Atlas MS20  (37-66”)

  - Atlas DS-7 (8-13”)

- BOOM ARMS:

  - Atlas PB77(31”)

  - Atlas DMS10E (14-26”)

  - Atlas PB21X(25-38”)

  - K&M KM211/1(18-30”)

- STAND/BOOM COMBINATIONS:

  - K&M KM210/91(35-61” stand, 18-30” boom)

  - K&M KM259 (20-28” stand, 18-30” boom)

  - K&M KM259/1 (11” stand, 20” boom)

  - K&M KM259/5 (18” stand, 28.5” boom)

  - K&M KM275 (34-61” stand, 33” boom)

  - K&M KM277 (33.5” stand, 33” boom)

# VIDEO / STREAMING OVERVIEW

VIDEO SERVICES AVAILABLE include:

- Video streaming and recording (<16 inputs)

- Hybrid/Virtual meetings (via Zoom or Teams)

- Webinars

- IMAG (Image Magnification)

- Video Playback

- Projection/Surface Mapping

VIDEO/STREAMING INVENTORY*

*Most video inventory is shared with multiple theatres in the building. Note what is dedicated to the space and what is used building-wide. Availability for building-wide inventory cannot be guaranteed and should be advanced with the Director of Production and/or Technical Coordinator. Additional rental fees may apply.

SCREENS

- 2 – 9’ x 16’ front projection screen

- 1 – 19’ x 25’ front/rear screen

- 1 – 26’ x 42” front projection screen (for surtitles)

PROJECTORS

  - 2 – Panasonic PT-MX16KBU77 – 16,000 lm; 4K resolution

  - 2 – Eiki LCHDT2000 – 15,000 lm; 2K resolution

MONITOR/TV SCREENS

- 2 – 48” TV screens with low latency for conductor cams (built into Playhouse, on balcony rail)

- 10 – 55” HD digital TV screens on rolling carts

- 2 – 65” HD digital TV screens on rolling carts

- 4 – 24” 4K confidence monitors

- 2 – 48” HD digital TV screens with low latency

- 4 – 32” SD analog TVs on rolling carts

CAMERAS

- 4 – Panasonic AW-HE42KPJ PTZ cameras

- 1 – HD PTZ cameras (built into Playhouse, under balcony)

- 3 – Sony DXC-390 3CCD Cameras w/ Canon YH19x6.7 KRS lens (as conductor cams)

VIDEO SWITCHERS

Full production switches come with a cart complete with multiple 4K monitors, Apple Mac Mini, audio mixer, and more. Mini switchers are stand-alone.

- 2 – Blackmagic ATEM 4K HD8 Streaming/Recording Desks

- 1 – Blackmagic ATEM 2M/E Constellation 4K Production Switcher

- 3 – Blackmagic ATEM Mini, HD video switchers

MISCELLANIOUS

- Multiple PC and Apple laptops loaded with all major presentation software

  - Ex. PowerPoint, Keynote, etc

- 4 – DScan Perfect Cue Presentation Remotes

# MEDIA ROOM

Overture Center is equipped with a media connection room, accessible via a cable pass-through to Henry Street.  This room provides Audio (XLR) and Video (BNC) connections to nine separate locations throughout the building.

- 2 – locations in the Overture Hall Lobby

- 3 – locations in Overture Hall (Broadcast booth, Projection booth and H.R. Mezzanine)

- 1 – located in the Capitol Theater (Sound booth)

- 1 – located in The Playhouse (House Left slit)

- 1 – located in Promenade Hall

- 1 – located in MMoCA Lecture Hall

Each location has three groups of connections.  Each group has two male and two female XLR audio connections, and two female BNC composite video connections.

# GROUNDPLAN – STAGE LEVEL

# GROUNDPLAN – RIGGING/LIGHTING

# GROUNDPLAN – SECTION VIEW

# PLAYHOUSE SEATING CHARTS

ORCHESTRA LEVEL

BALCONY LEVEL